MINOR PROJECT PROPOSAL · DEPARTMENT OF INFORMATION TECHNOLOGY · 2026

PROBLEM IN SIGNAL PROCESSING & DEEP LEARNING

Audiogram-Personalised Speech Enhancement
for Hearing Aids

Using deep learning to personalise noise reduction based on each individual's unique hearing loss profile

| Jwanil Modi 23BIT194 | Namya Shah 23BIT027 |
| --- | --- |

5-Model Comparison   |   HASPI / HASQI Evaluation   |   Mamba SOTA   |   November 1, 2026

THE PROBLEM

Hearing Aids Amplify. They Don't Understand.

| 466 Million people worldwide with disabling hearing loss (WHO) | #1 Complaint Understanding speech in noisy environments |
| --- | --- |

|  | THE CORE PROBLEM Modern hearing aids apply the same noise-reduction algorithm to every user — regardless of their individual hearing loss profile. |
| --- | --- |

|  | WHY THIS IS WRONG Every person's hearing loss is different. Person A loses high frequencies. Person B loses low frequencies. The same algorithm cannot be optimal for both. |
| --- | --- |

|  | OUR SOLUTION A neural network (a type of AI) that reads each person's audiogram (hearing test result) and personalises its noise-reduction behaviour specifically for that user. |
| --- | --- |

Three Different Audiogram Profiles

| Frequency → | 250 Hz | 500 Hz | 1000 Hz | 2000 Hz | 4000 Hz | 8000 Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Normal | 10 dB | 10 dB | 10 dB | 10 dB | 10 dB | 10 dB |
| HF (High-Freq) Loss | 10 dB | 15 dB | 20 dB | 45 dB | 70 dB | 85 dB |
| Flat Severe | 60 dB | 60 dB | 60 dB | 60 dB | 60 dB | 60 dB |

dB HL = decibels Hearing Level. Higher number = worse hearing at that frequency.

KEY CONCEPT

What Is an Audiogram?

An audiogram is the result of a clinical hearing test. It records the quietest sound a person can hear at 6 standard frequencies — from deep bass (250 Hz) to high treble (8000 Hz).

The 6 Test Frequencies

| Frequency | Sound Region | Speech Content |
| --- | --- | --- |
| 250 Hz | Deep bass | Low vowel sounds, background hum |
| 500 Hz | Bass | Vowels (a, o, u) |
| 1000 Hz | Mid | Vowels, general speech energy |
| 2000 Hz | Mid-high | Speech clarity, consonant transitions |
| 4000 Hz | High | Consonants (s, f, sh, ch) |
| 8000 Hz | Very high | Fricatives (s, f), air sounds |

Hearing Loss Classification

| Range (dB HL) | Category | Practical Impact |
| --- | --- | --- |
| 0–25 dB HL | Normal | Can hear whispers |
| 25–40 dB HL | Mild | Misses quiet speech |
| 40–70 dB HL | Moderate | Misses normal conversational speech |
| 70+ dB HL | Severe/Profound | Only hears loud sounds |

audiogram_vector = [10, 15, 20, 45, 70, 85] # dB HL at 6 frequencies normalised = [0.10, 0.15, 0.20, 0.45, 0.70, 0.85] # divided by 100 → model input

|  | WHY THIS IS OUR MODEL'S INPUT The audiogram tells us exactly which frequencies the person struggles with. Hearing aids already personalise amplification per audiogram (using the NAL-R prescription formula). We extend this to noise reduction — which is currently the same for every user. |
| --- | --- |

LITERATURE REVIEW & RESEARCH GAP

What Exists — And What Doesn't

What Already Exists

Paper 1: Diehl et al., 2023 (Nature Scientific Reports) — Generic Deep Neural Network denoising that restored intelligibility to near-normal levels. No audiogram conditioning.

Paper 2: Conv-TasNet (Luo & Mesgarani, 2019) — State-of-the-art waveform-domain separation. No personalisation.

Paper 3: SEMamba (Chao et al., 2024) — Current SOTA (State of the Art): PESQ (Perceptual Evaluation of Speech Quality) score 3.69. No hearing-aid awareness.

Challenge: Clarity Challenge (University of Salford, 2021–2024) — Hearing-aid-specific benchmark. Winning systems use audiograms for scoring, not for conditioning the model.

Paper 4: NeuroAMP (Ahmed et al., 2025, IEEE TAI) — Audiogram conditioning for amplification, NOT for noise reduction.

Technique: FiLM (Perez et al., 2017) — Feature-wise Linear Modulation conditioning mechanism. Originally for computer vision tasks.

The Gap We Target

|  | Gap 1 No published system uses an audiogram as a conditioning input to a noise-reduction neural network. Audiograms are used for amplification or post-hoc evaluation only. |
| --- | --- |

|  | Gap 2 State-of-the-art architectures (SEMamba, CMGAN) are not evaluated on hearing-aid-specific metrics (HASPI/HASQI) and ignore the <10 ms latency constraint real hearing aids require. |
| --- | --- |

|  | Our Specific Novelty Mamba/SSM + FiLM conditioning + MetricGAN+ trained with HASPI discriminator. No paper combines all three. The HASPI-GAN discriminator specifically is unpublished. |
| --- | --- |

OUR APPROACH

5-Model Comparison Architecture

| # | Model Name | Type | Key Feature | Audiogram? |
| --- | --- | --- | --- | --- |
| 1 | Wavelet DWT (Discrete Wavelet Transform) Denoiser | Classical | DWT soft-thresholding, non-stationary noise | No |
| 2 | MMSE-LSA (Minimum Mean Square Error – Log Spectral Amplitude) Filter | Classical | Industry standard in commercial hearing aids today | No |
| 3 | 1D CNN (1-Dimensional Convolutional Neural Network) / Conv-TasNet | Deep learning — generic | Waveform-domain, dilated convolutions | No |
| 4 ★ | U-Net + Self-Attention + FiLM (Feature-wise Linear Modulation) | Deep learning — personalised | CRM (Complex Ratio Mask) + MetricGAN+ with HASPI discriminator | YES |
| 5 | Mamba/SSM (State Space Model) + FiLM | Deep learning — personalised | SOTA accuracy (PESQ 3.69), linear complexity O(T) | YES |

|  | THE NARRATIVE Classical → Generic deep learning → Personalised deep learning → SOTA personalised. The key comparison: Models 4 & 5 vs Model 3 should show a larger HASPI improvement than STOI (Short-Time Objective Intelligibility) improvement — proving that audiogram conditioning specifically helps hearing-impaired listeners. |
| --- | --- |

MODELS 1 & 2

Classical Signal Processing Baselines

MODEL 1 — Wavelet DWT (Discrete Wavelet Transform) Denoising

Converts the audio signal into a wavelet representation — similar to a spectrogram but with adaptive time-frequency resolution. Noise appears as small coefficients everywhere; speech concentrates in large coefficients at specific scales. We apply soft-thresholding to remove small coefficients (noise) while keeping large ones (speech).

DWT(signal) → wavelet coefficients at levels 1..L threshold λ = σ × √(2 × log(N)) [σ = noise std estimated via MAD] soft_threshold(coeff) → IDWT → clean signal Wavelet: db8 (Daubechies-8), level 5. Library: PyWavelets (pywt)

MODEL 2 — MMSE-LSA (Minimum Mean Square Error – Log Spectral Amplitude) Filter

The algorithm actually running in commercial hearing aids today (Oticon, Signia, Phonak). Uses a "decision-directed" approach to estimate the signal-to-noise ratio frame-by-frame, then applies a perceptually-motivated gain that suppresses noise while preserving speech. No machine learning — pure signal processing. Virtually zero latency.

ξ(k,n) = α × Â²(k,n-1)/λ_n + (1-α) × max(γ-1, 0) α = 0.98, ξ = a priori SNR (Signal-to-Noise Ratio), γ = instantaneous SNR Reference: Ephraim & Malah (1985), IEEE TASLP

|  | WHY THESE BASELINES MATTER These are not toy baselines. MMSE-LSA is the industry standard — it runs in virtually every commercial hearing aid sold today. Beating it with deep learning is a meaningful result. (Note: The Wiener filter was excluded as too basic — faculty feedback.) |
| --- | --- |

MODEL 3

1D Convolutional Neural Network — Conv-TasNet Style

Works on the raw audio waveform directly — no STFT (Short-Time Fourier Transform) spectrogram needed. Learns its own internal frequency decomposition from data using 1D convolutions. Achieves very low latency. No audiogram input — serves as the "what does DL alone get us" baseline.

Architecture Block Diagram

| Stage | Operation | Output Shape | Purpose |
| --- | --- | --- | --- |
| Encoder | 1D Conv: kernel=16, stride=8, N=256 filters | (B, 256, T/8) | Learned STFT replacement |
| TCN Block 1 | Dilated Conv, dilation=1 | (B, 256, T/8) | Local context, 1-frame window |
| TCN Block 2–8 | Dilated Conv, dilation=2,4..128 | (B, 256, T/8) | Growing receptive field up to ~32 ms |
| Mask head | Linear → Sigmoid | (B, 256, T/8) | Mask values in [0,1] |
| Decoder | 1D ConvTranspose | (B, 1, T) | Reconstruct enhanced waveform |

Why Dilated Convolutions?

Standard 1D convolution with kernel=3 sees only 3 adjacent frames. To capture 1 second of context at 16 kHz you would need 500+ stacked layers. Dilation lets each layer skip frames, exponentially growing the receptive field with only 8 layers:

dilation=1: sees frames [t-1, t, t+1] — 3 frames dilation=8: sees frames [t-8, t, t+8] — 17 frames dilation=128: sees frames [t-128, t, t+128] — 257 frames → Total receptive field across 8 blocks: 511 frames ≈ 32 ms at 16 kHz

|  | ROLE IN OUR COMPARISON Model 3 answers: "What does a state-of-the-art generic DNN achieve without any personalisation?" Models 4 and 5 must beat this using the audiogram. The improvement (HASPI gain) = the measured benefit of personalisation. |
| --- | --- |

MODEL 4 — CORE CONTRIBUTION

U-Net + Self-Attention + FiLM Conditioning

Processing Pipeline

| Step | Operation | Input → Output |
| --- | --- | --- |
| 1 | STFT (Short-Time Fourier Transform) | Waveform (B, T) → Complex Spectrogram (B, F=257, T_frames) |
| 2 | Encoder — 4 × 2D CNN stages + stride=2 | (B, 1, F, T) → (B, 256, F/16, T/16) |
| 3 | Self-Attention in bottleneck | (B, 256, H, W) → same shape, global context added |
| 4 | FiLM Layer ← Audiogram vector [a₁..a₆] | γ × features + β (personalisation applied here) |
| 5 | Decoder — 4 × ConvTranspose + skip connections | (B, 256, H/16, W/16) → (B, 2, F, T) |
| 6 | CRM (Complex Ratio Mask) application | M_real, M_imag × Noisy complex spectrum |
| 7 | ISTFT (Inverse Short-Time Fourier Transform) | Enhanced complex spectrum → Enhanced waveform |

Three Stacked Innovations

|  | INNOVATION 1: FiLM Conditioning (Personalisation) A small auxiliary MLP (Multi-Layer Perceptron: 6→64→128→512) maps the audiogram vector to scale (γ) and shift (β) parameters. Applied to bottleneck features: output = γ × features + β. Different audiograms produce different γ and β → different internal processing for each user. |
| --- | --- |

|  | INNOVATION 2: Complex Ratio Mask (CRM) Predicts TWO mask channels — M_real and M_imag — and multiplies them with the complex spectrogram. This modifies both magnitude AND phase simultaneously, eliminating "musical noise" artifacts caused by the standard IRM (Ideal Ratio Mask) that only touches magnitude. |
| --- | --- |

|  | INNOVATION 3: MetricGAN+ with HASPI Discriminator ★ (Strongest Novelty) A discriminator network learns to predict HASPI (Hearing Aid Speech Perception Index) scores. The generator (our U-Net) trains adversarially to maximise predicted HASPI — directly optimising for hearing-impaired listener intelligibility. Standard MetricGAN+ uses PESQ; using HASPI is our specific unpublished contribution. |
| --- | --- |

FILM — FEATURE-WISE LINEAR MODULATION

How the Model Reads the Audiogram

FiLM is a conditioning mechanism that allows a neural network to modify its own internal behaviour based on a side input — in our case, the audiogram. Rather than concatenating the audiogram to the input (which doesn't work well for 2D feature maps), FiLM generates per-channel scale-and-shift parameters that directly modulate the feature maps at the bottleneck.

FiLM Operation

audiogram a = [a₁, a₂, a₃, a₄, a₅, a₆] ← 6 normalised hearing thresholds FiLM Generator (small MLP): a (6-dim) → Linear(64) → ReLU → Linear(128) → ReLU → Linear(512) → γ (256) + β (256) FiLM Application: output(b, c, h, w) = γ(b, c) × features(b, c, h, w) + β(b, c) (γ and β broadcast over spatial dimensions h and w)

What FiLM Learns in Practice

| Audiogram Type | Expected FiLM Behaviour | Effect |
| --- | --- | --- |
| Severe High-Freq Loss [10,15,20,45,70,85] | γ/β amplify high-freq noise-detection channels | More aggressive HF noise removal — person can't hear HF anyway |
| Flat Moderate Loss [40,40,40,40,40,40] | γ/β adapt uniformly across all channels | Balanced enhancement across all frequencies |
| Near-Normal [10,10,10,10,10,10] | γ ≈ 1, β ≈ 0 (near-identity) | Minimal modification — person barely needs personalisation |

Critical Verification Test

# Run the same noisy clip with two different audiograms mask_normal = model(noisy, audiogram=[10,10,10,10,10,10]) mask_hf = model(noisy, audiogram=[10,15,20,45,70,85]) # MUST be numerically different — if identical, FiLM is not being used assert torch.max(torch.abs(mask_normal - mask_hf)) > 0.01 print("✅ Personalisation verified")

MODEL 5 — STATE-OF-THE-ART

Mamba / State Space Model (SSM) + FiLM

Why Mamba Beats Transformers for Audio Processing

| Property | Standard Transformer | Mamba / SSM | Why It Matters for Hearing Aids |
| --- | --- | --- | --- |
| Complexity | O(T²) — quadratic | O(T) — linear | Hearing aids process continuous audio; quadratic cost explodes with time |
| Real-time mode | Requires future masking hack | Naturally causal (left→right) | Real-time = cannot look at future frames |
| Best PESQ score | ~3.40 | 3.69 (SEMamba, 2024) | More accurate AND faster — not a tradeoff |
| Parameter count | 10–30M typical | 2–5M | Smaller models fit on hearing aid hardware |

What Is a State Space Model (SSM)?

A State Space Model describes a system with memory: the current output depends on everything that happened before, compressed into a hidden state h(t).

Standard SSM (fixed matrices): h(t) = A × h(t-1) + B × x(t) ← hidden state update y(t) = C × h(t) ← output Mamba (selective — matrices are input-dependent): A(t), B(t), C(t) = f(x(t)) ← computed from current input! h(t) = A(t) × h(t-1) + B(t) × x(t) ← selective memory y(t) = C(t) × h(t) ← selective output → When it hears a speech onset: opens gate, strongly updates h → When it hears steady noise: closes gate, barely updates h

|  | SIMPLE ANALOGY An old RNN (Recurrent Neural Network) is like a conveyor belt that moves everything forward at the same speed. Mamba is like a smart gatekeeper who decides what gets remembered and what gets discarded — based on what is currently being processed. |
| --- | --- |

|  | OUR ADDITION: FiLM CONDITIONING SEMamba (2024) has no audiogram input. We add a FiLM layer after the Mamba blocks. This personalises the SSM's processing per audiogram. The Mamba+FiLM combination for hearing aids is not published. Reference: Chao et al., 2024 (SEMamba). |
| --- | --- |

SYSTEM ARCHITECTURE

Complete Pipeline — Block by Block

The system has two inputs: (1) noisy speech waveform from the hearing aid microphone, and (2) the user's audiogram vector. The output is enhanced speech personalised for that user.

| Block | Operation | Input Shape | Output Shape | Notes |
| --- | --- | --- | --- | --- |
| INPUT A | Noisy waveform from mic | (B, T) samples | (B, T) | 16 kHz, 10ms frames |
| INPUT B | Audiogram vector | (B, 6) dB HL values | (B, 6) normalised | Normalised to [0, 1] |
| STFT | Short-Time Fourier Transform | (B, T) | (B, F=257, T_frames) complex | Window=512, hop=128 |
| Encoder | 4× 2D Conv stages + BatchNorm + LeakyReLU | (B, 1, 257, T) | (B, 256, 16, T/16) | Each stage: stride=2 |
| Self-Attention | Q/K/V attention over flattened spatial dims | (B, 256, H, W) | (B, 256, H, W) | Global context, gamma=0 init |
| FiLM | Audiogram MLP → γ, β → scale+shift features | (B, 256, H, W) + (B,6) | (B, 256, H, W) | Personalisation step |
| Decoder | 4× ConvTranspose + skip connections from encoder | (B, 256, H, W) | (B, 2, 257, T) | 2 channels = M_real, M_imag |
| CRM Apply | (M_real + j×M_imag) × (Y_real + j×Y_imag) | (B, 2, F, T) + noisy spec | (B, F, T) complex | Phase-aware masking |
| ISTFT | Inverse Short-Time Fourier Transform | (B, F, T) complex | (B, T) waveform | Enhanced speech |

DATA

Datasets and Data Strategy

| Dataset | Role | Size | Key Feature |
| --- | --- | --- | --- |
| TIMIT | Clean speech — training | 6,300 utterances | Phoneme labels, balanced speakers |
| NOIZEUS | Evaluation benchmark | 30 sentences × 8 noises × 4 SNRs | Standard SE (Speech Enhancement) evaluation corpus |
| Clarity CEC2/3 | Primary HA (Hearing Aid) training | ~11,000 scenes | Real listener audiograms included, purpose-built for HA research |
| VoiceBank-DEMAND | Secondary / sanity check | 11,572 utterances | Standard SE benchmark — for cross-paper comparison |
| MUSAN + ESC-50 | Noise augmentation | 900+ noise clips | Diverse real-world noise types for dynamic mixing |

Dynamic Mixing Strategy (Training)

# Instead of pre-creating fixed noisy files, mix on-the-fly every epoch: SNR = random.uniform(-5, +10) # dB, drawn fresh each step noise = random.choice(MUSAN) # random noise file noisy = clean + 10^(-SNR/20) × noise # → Effective infinite training data. Model never memorises a fixed noise file.

|  | AUDIOGRAM STRATEGY FOR TIMIT-BASED TRAINING TIMIT has no audiograms. We sample synthetic audiograms from a realistic statistical distribution based on WHO/clinical hearing loss prevalence data. Clarity Challenge provides real listener audiograms for training on HA-specific data. |
| --- | --- |

EVALUATION

Metrics — Why Standard Metrics Aren't Enough

Hearing-Aid Specific Metrics (Our Primary Metrics)

| Metric | Full Name | Takes Audiogram? | Range | What It Measures |
| --- | --- | --- | --- | --- |
| HASPI | Hearing Aid Speech Perception Index | YES ★ | 0→1 | Predicted intelligibility for hearing-impaired listener |
| HASQI | Hearing Aid Speech Quality Index | YES ★ | 0→1 | Predicted quality (naturalness) for hearing-impaired listener |

Standard Reference Metrics

| Metric | Full Name | Audiogram? | Range | Purpose |
| --- | --- | --- | --- | --- |
| STOI | Short-Time Objective Intelligibility | No | 0→1 | Intelligibility for normal-hearing listeners |
| SI-SDR | Scale-Invariant Signal-to-Distortion Ratio | No | dB (higher = better) | Signal quality, used as training loss |
| PESQ | Perceptual Evaluation of Speech Quality | No | 1→4.5 | Quality metric — used in SE literature for cross-paper comparison |
| Latency | Processing delay per 10 ms frame | No | ms (lower = better) | Must be <10 ms for real-time hearing aid deployment |

|  | THE KEY SCIENTIFIC RESULT WE EXPECT For Models 4 and 5 vs Model 3 (generic DNN): ΔHASPI >> ΔSTOI Meaning: the personalised model helps hearing-impaired listeners MORE than it helps general listeners. This proves audiogram conditioning provides hearing-specific benefit, not just generic noise removal. |
| --- | --- |

EXPECTED OUTCOMES

Results We Anticipate

Expected Performance on NOIZEUS Benchmark

| Model | HASPI ↑ | PESQ ↑ | STOI ↑ | Latency ↓ | Params |
| --- | --- | --- | --- | --- | --- |
| Noisy (no processing) | ~0.30 | ~1.6 | ~0.65 | 0 ms | — |
| Model 1: Wavelet DWT | ~0.45 | ~2.0 | ~0.74 | <1 ms | — |
| Model 2: MMSE-LSA | ~0.55 | ~2.4 | ~0.78 | <1 ms | — |
| Model 3: 1D CNN | ~0.65 | ~3.0 | ~0.86 | ~3 ms | ~5M |
| Model 4: U-Net+FiLM ★ | ~0.75 | ~3.3 | ~0.89 | ~8 ms | ~15M |
| Model 5: Mamba+FiLM | ~0.80 | ~3.6 | ~0.91 | ~6 ms | ~5M |

* HASPI values are for the severe HF loss audiogram [10,15,20,45,70,85]. Numbers are informed estimates from published literature, not guaranteed results.

Three Test Audiogram Profiles

| Profile ID | Values (dB HL at 250–8000 Hz) | Description | Expected personalisation gain |
| --- | --- | --- | --- |
| Profile A | [10, 10, 10, 10, 10, 10] | Normal / near-normal hearing | Smallest — model barely needs to adapt |
| Profile B | [10, 15, 20, 45, 70, 85] | Severe high-frequency loss (most common) | Largest — strong HF suppression learned |
| Profile C | [60, 60, 60, 60, 60, 60] | Flat severe loss | Moderate — uniform adaptation across frequencies |

PROJECT TIMELINE

August 2026 → November 1, 2026 (77 Days)

| Phase | Name | Dates | Lead | Status |
| --- | --- | --- | --- | --- |
| 0 | DSP (Digital Signal Processing) Fundamentals | Aug 15–22 | Both | 🟡 In Progress |
| 1 | Audiology + Audiogram Generation | Aug 23–29 | Both | ⬜ TODO |
| 2 | Classical Baselines: Wavelet DWT + MMSE-LSA | Aug 30–Sep 6 | Jwanil | ⬜ TODO |
| 3 | Data Pipeline (TIMIT + Clarity + NOIZEUS) | Sep 7–20 | Namya | ⬜ TODO |
| 4 | 1D CNN Model (Conv-TasNet) | Sep 21–Oct 4 | Both | ⬜ TODO |
| 5 | U-Net + Attention + FiLM (Core Contribution) | Oct 5–18 | Both | ⬜ TODO |
| 5b | Mamba/SSM + FiLM (SOTA Model) | Oct 19–25 | Jwanil | ⬜ TODO |
| 6 | Full Evaluation — all 5 models, all metrics | Oct 26–28 | Both | ⬜ TODO |
| 7 | Report + Audio Demo | Oct 29–31 | Both | ⬜ TODO |
| 🎯 | SUBMIT | November 1, 2026 | — | 🎯 DEADLINE |

IMPLEMENTATION

Tools, Libraries & Software Stack

| Library | Version | Purpose | Install |
| --- | --- | --- | --- |
| PyTorch | ≥2.1 | Core deep learning framework | pip install torch torchaudio |
| Asteroid | ≥0.6 | Conv-TasNet, SI-SDR loss, source sep tools | pip install asteroid |
| SpeechBrain | ≥1.0 | MetricGAN+ reference implementation | pip install speechbrain |
| mamba-ssm | ≥1.0 | Mamba State Space Model layers | pip install mamba-ssm |
| pyclarity | ≥0.4 | HASPI & HASQI metrics, HA simulation | pip install pyclarity |
| pystoi | ≥0.4 | STOI metric | pip install pystoi |
| pesq | ≥0.0.4 | PESQ metric | pip install pesq |
| PyWavelets | ≥1.4 | DWT denoising (Model 1) | pip install PyWavelets |
| librosa | ≥0.10 | Audio analysis & visualisation | pip install librosa |
| TensorBoard | ≥2.14 | Training loss & metric monitoring | pip install tensorboard |

pip install torch torchaudio asteroid speechbrain pyclarity pystoi pesq PyWavelets mamba-ssm librosa matplotlib

NOVELTY & CONTRIBUTION

What Makes This Project Original

|  | CLAIM 1: Audiogram-Conditioned Noise Reduction NeuroAMP (2025) uses audiograms for amplification. No published system uses audiogram conditioning for the noise reduction step. We are the first to apply FiLM to hearing-aid noise reduction specifically. |
| --- | --- |

|  | CLAIM 2 ★: HASPI-Optimised GAN Training — Strongest Claim MetricGAN+ trains a discriminator on PESQ (Perceptual Evaluation of Speech Quality). We train our discriminator on HASPI (Hearing Aid Speech Perception Index) — which takes the audiogram as input and measures intelligibility for hearing-impaired listeners. This specific combination has not been published anywhere. |
| --- | --- |

|  | CLAIM 3: Mamba/SSM + FiLM for Hearing Aids SEMamba (Chao et al., 2024) achieves SOTA PESQ 3.69 but has no audiogram conditioning. We add FiLM to a SEMamba-style architecture for personalised hearing aid enhancement. The combination is novel. |
| --- | --- |

|  | CLAIM 4: Systematic 5-Model Comparison with HA-Specific Metrics A rigorous comparison from classical DSP through SOTA deep learning, evaluated on HASPI/HASQI across multiple audiogram profiles, has not been published in this form. Directly useful to the field regardless of the primary novel results. |
| --- | --- |

FUTURE SCOPE

Where This Goes Next

|  | 🔬 Real Hardware Testing Deploy the Mamba+FiLM model on actual hearing aid DSP (Digital Signal Processor) chips — e.g. Arm Cortex-M55 + Ethos-U55 NPU (Neural Processing Unit). Benchmark true on-device latency, power draw, and memory footprint. Validate that the <10ms constraint holds in silicon, not just simulation. |
| --- | --- |

|  | 📄 Research Paper Submission The HASPI-optimised MetricGAN+ discriminator + audiogram FiLM conditioning is an original contribution. Target venues: ICASSP 2027 (IEEE International Conference on Acoustics, Speech and Signal Processing), Interspeech 2027, or IEEE/ACM TASLP (Transactions on Audio, Speech, and Language Processing). Deadline for ICASSP 2027: ~September 2026. |
| --- | --- |

|  | 🌐 Indian Language Testing Evaluate robustness on Indian languages — Hindi, Gujarati, Bengali, Tamil — using IndicSpeech or MUCS (Multilingual and Code-switching ASR) datasets. Indian speech has different phoneme inventories and retroflex consonants. This is a genuinely underexplored gap in the hearing-aid literature, and directly relevant to India's 63M+ hearing-impaired population. |
| --- | --- |

|  | 🔐 Federated Learning Audiograms are private medical data. Federated Learning (FL) trains the model across many hearing aid users without sending raw audio or audiograms to a central server — each device trains locally and shares only gradient updates. Privacy-preserving personalisation at scale. |
| --- | --- |

|  | 📱 Companion Mobile App A smartphone app where users input or import their audiogram (photo of their audiogram chart). The app connects to the hearing aid via Bluetooth LE, uploads the audiogram vector, and the model re-conditions itself in real time. Audiogram update = instant personalisation with no clinic visit required. |
| --- | --- |

|  | 🏥 Clinical User Study Recruit 20–30 hearing-impaired participants, play enhanced audio via calibrated headphones, collect subjective intelligibility and quality ratings. Compare our system vs commercial hearing aids. This is the gold standard validation — aligns with the Clarity Challenge's listener evaluation methodology (MOS: Mean Opinion Score). |
| --- | --- |

|  | ★ THE HIGHEST-IMPACT NEXT STEP Submit a paper to ICASSP 2027 on the HASPI-optimised MetricGAN+ finding. This is the specific result that has not been published — and it is directly testable from the deliverables of this minor project. |
| --- | --- |

CONCLUSION

Summary & Why This Project Matters

|  | THE PROBLEM IN ONE LINE 466 million hearing aid users get generic noise reduction. We personalise it per audiogram. |
| --- | --- |

|  | OUR APPROACH 5-model comparison from classical baselines to SOTA deep learning. FiLM conditioning makes the model "read" each person's audiogram and adapt its noise-removal behaviour specifically for them. |
| --- | --- |

|  | THE SCIENTIFIC CLAIM HASPI improvement from audiogram conditioning will be larger than STOI improvement — proving personalisation specifically benefits hearing-impaired listeners, not just audio quality in general. |
| --- | --- |

Deliverables by November 1, 2026

5 trained and evaluated models with documented metrics (HASPI, HASQI, STOI, SI-SDR, PESQ, latency)

Results table and 4 publication-quality plots (bar charts, scatter plots)

Audio demonstrations: before/after .wav files for 3 noise types × 3 audiogram profiles

Full written academic report (.docx and PDF)

Reproducible open-source code on GitHub (github.com/Jwanil)

Possibility of ICASSP 2027 submission for the HASPI-GAN finding

REFERENCES

Key Literature

| # | Authors | Venue | Year | Title / Notes |
| --- | --- | --- | --- | --- |
| 1 | Diehl et al. | Nature Scientific Reports | 2023 | "Restoring speech intelligibility for hearing aid users with deep learning" |
| 2 | Perez et al. | AAAI | 2018 | "FiLM: Visual Reasoning with a General Conditioning Layer" |
| 3 | Chao et al. | arXiv | 2024 | "SEMamba: State Space Model for Speech Enhancement" — PESQ 3.69 SOTA |
| 4 | Kates & Arehart | JASA | 2021 | "HASPI v2 / HASQI v2 — hearing aid intelligibility/quality indices" |
| 5 | Ahmed et al. | IEEE TAI | 2025 | "NeuroAMP: End-to-end Deep Neural Amplifier for Personalized Hearing Aids" |
| 6 | Luo & Mesgarani | IEEE/ACM TASLP | 2019 | "Conv-TasNet: Surpassing Ideal Time-Frequency Masking for Speech Separation" |
| 7 | Cao et al. | Interspeech | 2022 | "CMGAN: Conformer-based MetricGAN for Speech Enhancement" |
| 8 | Fu et al. | Interspeech | 2021 | "MetricGAN+: An Improved Version of MetricGAN for Speech Enhancement" |
| 9 | Hu et al. | Interspeech | 2020 | "DCCRN: Deep Complex Convolution Recurrent Network for Phase-Aware SE" |
| 10 | Gu & Dao | NeurIPS | 2023 | "Mamba: Linear-Time Sequence Modeling with Selective State Spaces" |
| 11 | Clarity Challenge | Univ. Salford / CVSSP | 2021–2024 | Clarity Enhancement Challenge series (CEC1, CEC2, CEC3) |
| 12 | Ephraim & Malah | IEEE TASLP | 1985 | "Speech enhancement using a minimum mean-square error log-spectral amplitude estimator" |

Thank You We welcome your questions, feedback, and guidance. Jwanil Modi 23BIT194 ¦ Namya Shah 23BIT027 github.com/Jwanil · All code will be open source

Minor Project Proposal · Department of Information Technology · 2026